VICI Notes to Data

# 1. Parameters

## 1.1 Given Parameters

- Layer number: Value indicates how many layer the cell has

- Ampere (A): Ampere applied on the cell (40.0 A)

## 1.2 Measured parameters

- LEL (%): Low Explosive Limit (or Lower Flammable Limit)

- Gas flow (mln / min): Gas flow in millilitres per minute (“n” stands for normal conditions → 0°C, 1013 mbar)

- Membrane V at 13 bars (V): Voltage at the membrane while pressurized

- Cell V at 13 bars (V): Voltage in the cell while pressurized

- Cell V at 0 bar (V): Voltage in the cell while not pressurized

## 1.3 Calculated parameters

- Voltage Difference (V): Average voltage increases per individual cell layer achieved by pressurization from 0 to 13 bar

  - Condition: =WENN(J2=0;"";WENN(ODER(J2<0;J2>0.03);"X";""))

  - Meaning: Fails if “Voltage Difference” is smaller than 0 or larger than 0.03

- H2 Production (SmL / A): Standard Millilitres of Hydrogen produced per Ampere

  - Condition: =WENN(L2=0;"";WENN(ODER(L2<7;L2>7.8);"X";""))

  - Meaning: Fails if “H2 Production” is smaller than 7 or larger than 7.8

## 1.4 Additional columns

- Job nr.

- Cell ID

- Membrane batch FA

- Test station nr.

- Cell type

- Errors

- Notes (date and time of test)

- Notes 1

- Notes 2

- Initials (who tested it)